APPENDIX F-2. Using HEC-HMS to finish up the model

Once the model is prepared from ArcGIS using the HEC-GeoHMS, we need to make some changes to the HEC-HMS model prepared

1). For SCS Loss Method:

- Salt Lake Traid  (GHCN) for Oct2010-Sep2011 with base flow

In this simulation, basic simulation concept is same as of Green and Ampt loss method for KF_C, Iutah site except some input parameter for sub-basin, precipitation and USGS observed data are different as follow.

- Sub-basin

- SCS curve method is selected as Loss method

- Under the loss title, corresponding initial abstraction, curve number and impervious percentage values have been entered for each sub basins.

- Lag time (min) under the title “Transform” which are same as of KF_C, Iutah site for each sub basin have been entered.

- Corresponding calculated base flow according to USGS, Fort Douglas (10172200) for Oct 2010 - Sep 2011 for each sub- basin have been entered.

- Precipitation data

- Under the time-window title, precipitation data from Oct 2010 to Sep 2011 has been entered.

- Under the table title, precipitation of Salt Lake Traid, GHCN for Oct 2010 - Sep 2011 in mm (Precip Gage 1) has been entered

- For all sub-basin precip gage 1 data has been entered.

- Similarly USGS, Fort Douglas (10172200) station observed discharge (cfs) value (gage 1) for Oct 2010 – Sep 2011 has been entered.

- Finally at outlet 1 USGS, observed value has been set and run the model in order to compare the simulated and observed flow.

For Green and Ampt

- KF_C, Iutah site precipitation station for 2015 with baseflow

After developing model following sub-basin, reach, precipitation, discharge and control specification parameters have been entered.

- Sub-basin

For all seven sub basins following parameter have been entered. Here, sub-basin W80 is presented as example.

Under sub-basin title

- For each sub-basin corresponding area is entered in square miles.

- For each sub-basin Green and Ampt loss method, SCS Unit Hydrograph as Transform Method, and Constant monthly Base flow method have been selected.

Under Loss title

- For each sub-basin corresponding initial content, saturation content, suction, Conductivity and impervious values have been entered.

Under Transform title.

- For each sub-basin corresponding Lag time (min) have been entered

Under Base Flow title

- For each sub-basin corresponding calculated values of base flow according to USGS, Fort Douglas (10172200) station for  a year 2015 have been entered.

After entering all the parameters shown above under the sub-basin, loss, transform and base flow for seven sub-basin (W80, W90, W100, W110, W120, W130 and W140), Parameter required for reach have been entered.

- Reach

For all there reaches (R30, R60 and R70) Muskingham routing method have been selected with value of muskingham coefficient K and X as 0.6 hour and 0.2. Here, reach R30 have been presented as example.

Under reach title

  - For all reach muskigham routing method have been selected

Under routing method

  - For all reaches the value of K and X entered as 0.6 hour and 0.2

- Precipitation

After all, the precipitation gage 1 have been created under time-series data and then following input data have been entered.

Under Time-series Gate title

  - Precipitation data have been entered in “millimeter” under the unit

  - Precipitation data have been entered at time-step of “1 day” under time interval.

Under Time-Window table

- Start date, start time, end date and end time has been entered for 2015 precipitation data of KF_C iUtah site.

Under table title

- Precipitation data of 365 days of a year 2015 in millimeter under Precip Gage 1 has been entered manually.

- The Hyetograph of a year 2015 under KF_C iutah site can be seen below.

- The Metrologic model have been created and sub-basin are included by selecting “Yes” under the title Basins.

- Similarly Precipitation gage 1 has been set to all sub basins under the meteorological model and specified hyetograph

- Discharge

Under the Time-series, discharge Gage 1 has been created and discharge data (cfs) of USGS, Fort Douglas (10172200) with time-step 1 day has been entered in following ways.

Under the Time-series gate title

- Unit of discharge data has been set in “Cubic Feet per Second” under the unit section.

- The time-step has been set as “1 day” under the time interval section.

Under the Time window title

- Time period of observed discharge value of station (10172200)  have been set from 01Jan2015 to 31Dec2015.

Under the table title

- Observed discharge value at USGS, Fort Douglas station (10172200) for a year of 2015 at daily time step has been entered manually.

- The Hydrograph for a year of 2015 at USGS station (10172200) can be seen below.

- After all, observed discharge value (gage 1) set to outlet-1 in order to compare the simulated and observed discharged value at outlet-1 as shown in below.

- Similarly, simulation period has been controlled with period from 01Jan2015 to 01Dec2016 and 1 day time-step.

- Finally the model was run to compare the simulated and observed value at outlet-1.

2). Green and Ampt Loss Method:

- ARBR_C, TM_C and KF_C Iutah site (three-precipitation) station for 2015 with base-flow.

In this simulation, each and every steps are same as for KF_C , Iutah site except three precipitation stations has been considered based on elevation and location of sub-basins and stations as follow.

  - For sub-basin W80, W90, W100 and W120 : KF_C, Iutah precipitation station data has been entered

  - For sub-basins W110 and W130: TM_C, Iutah precipitation data has been entered.

  - For sub-basin W140 : ARBR_C, Iutah precipitation station data has been entered.

- Precipitation gage 1 (ARBR_C, Iutah) data for sub basin W140 in “mm”

- Precipitation gage 2 (TM_C, Iutah) data for sub basin W110 and W130 in “mm”

- Precipitation gage 2 (KF_C, Iutah) data for sub basin W80, W90, W100 and W120 in “mm”